Report For Lab 5

Connor McCullough

EEN 312

18 March 2013

Objective:

Write two different programs that turns on different LED lights for periods of time by using the timer registers.  This requires understanding how the timer in the 8051 functions.

Equipment:

Software:

- Text editor and 8051 ASM assembler

- Step debugger to execute program one instruction at a time

- Data memory, accumulator, code memory

- Built in LED lights

Results:

Part A: LED 0 is on for 1 second.

Part A: LED 1 is on for 1.5 seconds

Part A: LED 2 is on for 2 seconds

Part A: Sequence repeats indefinitely

Part B: Light is off for one initially

Part B: Light complimented after one second.

Part B: Light complimented again after 2 seconds

Part B: Light complimented again after 1 second.

Conclusion:

This lab taught the importance of understanding how timers work in the 8051 so that operations can occur at normal intervals.  The 8051 can’t just be told to wait a certain time, it has to loop until the timer has gone through a certain number of cycles.  This requires knowledge of how long each cycle takes in order to calculate the number of times the timer has to loop.

Code:

Lab 5a

Org 0h
clr p0.7
mov TMOD, #01 ; Timer mode 1

next1:
clr P1.0  ; Turn on LED 0
mov r5, #20 ; Timer will be for 1 second based on this register

again1:
mov TL0, #0B0h
mov TH0, #3Ch

setb TR0

here1:
jnb TF0, here1
clr TR0
clr TF0
dec r5
mov A, r5
jnz again1 ; Do again loop 20 times by decrementing each time

setb P1.0 ; Turn off LED 0

clr P1.1 ; Turn on LED 1

next2:
mov r5, #30 ; Timer will go for 1.5 seconds

again2:
mov TL0, #0B0h
mov TH0, #3Ch

setb TR0

here2:
jnb TF0, here2
clr TR0
clr TF0
dec r5
mov A, r5
jnz again2

setb P1.1 ; Turn off LED 1

clr P1.2 ; Turn on LED 2

next3:
mov r5, #60 ; Timer will go for 3 seconds
again3:
mov TL0, #0B0h
mov TH0, #3Ch

setb TR0

here3:
jnb TF0, here3
clr TR0
clr TF0
dec r5
mov A, r5
jnz again3
setb P1.2 ; Turn off LED 2
LJMP next1 ; Start over at end
End

Lab 5b

Org 0h
clr p0.7
mov TMOD, #01
next1:
mov R5, #20 ; Timer will be for 1 second

again1:
mov TL0, #0B0h
mov TH0, #3Ch
setb Tr0

here1:
JNB TF0, here1
clr TR0
clr TF0
dec R5
mov A, R5
JNZ again1
cpl p1.6 ; Compliment P1.6

next2:
mov R5, #40 ; Timer will be for two seconds

again2:
mov TL0, #0B0h
mov TH0, #3Ch
setb Tr0

here2:
JNB TF0, here2
clr TR0
clr TF0
dec R5
mov A, R5
JNZ again2
cpl p1.6
ljmp next1 ; Keep doing indefinitely
End